Table 1. Samples and techniques used for the total sulfur determination at trace levels.

| Sample | Detection Limit | Measurement Technique | Reference |
| --- | --- | --- | --- |
| Gasoline, petroleum Natural gas Naphtha | 0.1 mg l-1 0.5 mg m-3 8.9-9.6 mg m-3 0.1 ppm <20 ppb | Pyrolysis-Fluorimetry Combustion method (Combustion-SO42- titrimetry) Analyzer-Dual flame FPD Reductor (Raney Ni)-S2- titration Thermal cracking, reduction (Hydrogenation- H2S photometry) | (1) (2) (3) (4) |
| Organic matter Vegetable oils Plants Biological material Fats and oils Waste oils Cosmetic and pharmaceutical dyes | 0.001% 2-25 ng ml-1 0.05 mg L-1* 40 ng S (methionine) 60 ng S (cystine) 60 ng S (cysteine) 2 mg kg-1 0.07 mg kg-1 2 ng ml-1 SO42- * | Bomb Combustion-IC- SO42- conductimetry Reductor (Raney Ni)-S2- voltammetry Digestion-ICP-AES Pyrolysis-GC-FPD XRF ICP-AES Schöniger combustion-IC-conductimetry | (5) (6) (7) (8) (9) (10) (11) |
| Wine | 0.1 mg l-1 | Digestion-ICP-AES | (12) |
| Iron and steel Metals | 2 g g-1 280 ng 0.002%** 0.1 g | Digestion-H2S distillation-ID-ICP-MS Digestion-HG-cold trapping-GC-photo-ionization detector # Combustion-IR Combustion-IR | (13) (14) (15) (16) |

* in the measurement solution

** lowest level of the dynamic range